Zonal Office MUMBAI, G.A.D. Department

UCO BANK BUILDING, D.N.ROAD, FORT MUMBAI

Email-zomumbai.sec@ucobank.co.in

Website http://www.ucobank.com

Tendor Ref. No. – 2022/01, dated 10.12.2022

Corrigendum

Additional specification

Offer FOR EMPANELMENT OF VENDER FOR SUPPLY and INSTALLATION of CCTV eLECTRONIC Alarm systems (EAS) AND COMPREHENSIVE ANNUAL MAINTENANCE CONTRACT (CAMC) THEREOF in THE BANK BRANCHES UNDER ZONAL OFFICE, MUMBAI.

Vendor has to provide demonstrate of both old specification as well as new specification (Additional Specification)

## Additional Technical Specification & compliance for vendor:

## 4 Channel 2 SATA DVR Technical Specifications:

|  | Particulars | Specifications |
| --- | --- | --- |
| 1 | Operating system | Embedded Linux |
| 2 | CPU | Embedded Processor |
| 3 | Compatibility (Mandatory) | 4 Analog HD Cameras (Mandatory) /CVBS,AHD,TVI,CVI,IP |
| 4 | Video surveillance Hard disk | Surveillance Grade HD DVR with Hard Disk |
| 5 | System Resources | Pentaplex function: recording playback, live view, backup, remote access over IP network |
| 6 | Recorder Control Options | IP Network, USB mouse |
|  | Ports | 2 SATA Ports For 4 Channel HD DVR |
| 7 |  | Each SATA Port Should Support 10 TB HDD or more |
| 8 | Interface | 1 HDMI, 1 VGA, |
| 9 | Encoding Resolution | 5MP, 4MP, 3MP, 1080P, 720P, 960H, D1, CIF, QCIF |
| 10 | IP Camera input | 4+2 channels, each channel up to 6MP |
| 11 | Video Input | 4 Ch PAL (NTSC to PAL Conversion not acceptable) |
| 12 | Display speed | 100 fps for 4 channel (25 fps for each channel) or more |
| 13 | Compression method | AI Coding/H.265+/H.265/H.264+/H.264 |
| 14 | Audio In/Out | 4 audio input and 1 audio output |
| 15 | Recording Resolution | 5M-N/4M-N/1080P/1080N/720P/960H/ D1/CIF |
| 16 | Recording rate | 5M-N (1 fps–10 fps); 4M-N/1080p (1 fps–15 fps); 1080N/720p/960H/ D1/CIF (1 fps–25/30 fps) |
| 17 | Recording Interval Prerecord: | 1sec to 30 sec., Post-record: 10sec ~ 5 minutes |
| 18 | Video Encoding Bit Rate Analog video: | 32 kbps–6144 kbps per channel |
| 19 | Image Quality | 6 levels (VBR/ CBR) |
| 20 | Independent Recording setting Per channel | Manual; Schedule (General, Continuous); MD (Video detection: Motion Detection, Video Loss, Tampering); Alarm; Stop |
| 21 | Monitoring | 4 channel real time Monitoring |
|  |  | Single mode & Quad modes |
|  |  | Sequential switching mode |
|  |  | Switching out monitoring |
| 22 | Searching/Playback | Date & Time based |
|  |  | 1 channel searching and all channel simultaneous searching |
|  |  | Time/Date, Alarm, MD and Exact Search (accurate to second) |
| 23 | Object classification | Human/Vehicle secondary recognition for tripwire and intrusion |
| 24 | Smart Motion detection | Up to 4 channels video stream ( analog channel ) SMD Plus |
| 25 | Perimeter Protection | Up to 1 channel video stream ( analog channel ) Perimeter Protection |
| 26 | Back Up facility | USB Device/Network |
| 27 | Network Interface | 1 RJ-45 Port (1000M) |
| 28 | User Interface | On screen graphic User Interface(GUI) |
| 29 | System recovery after power failure | Auto-Rebooting |
| 30 | Alarm input | 8 Alarm Inputs |
| 31 | Alarm Output | 3 Alarm Ouput |
| 32 | USB Interface Minimum | 2 USB ports (USB 2.0) |
| 33 | Serial Interface | RS485 1 port, for PTZ control |
| 34 | Playback Function | Play; Pause; Stop; Rewind; Fast play; Slow Play; Next File; Previous File; Next Camera; Previous Camera; Full Screen; Repeat; Shuffle; Backup Selection; Digital Zoom |
| 35 | Search Mode | Time/Date, Alarm, MD and Exact Search (accurate to second) |
| 36 | Video Alarm Type | Recording, PTZ, Tour, Video Push, Email, FTP, Snapshot, Buzzer and Screen Tips, Motion Detection, Video Loss, Tampering and Diagnosis |
| 37 | Operating Conditions | -10°C ~ +55°C (+14°F ~ +131°F), 0~ 90% RH |
| 38 | Certifications | CE/ FCC/ UL and BIS certification |

8 Channel 4 SATA DVR Technical Specifications:

| Sr. No. | Particulars | Specifications |
| --- | --- | --- |
| 1 | Operating system | Embedded Linux |
| 2 | CPU | Embedded Processor |
| 3 | Compatibility (Mandatory) | 8 Analog HD Cameras (Mandatory) /CVBS,AHD,TVI,CVI,IP |
| 4 | Video surveillance Hard disk | Surveillance Grade HD DVR with Hard Disk |
| 5 | System Resources | Pentaplex function: recording playback, live view, backup, remote access over IP network |
| 6 | Recorder Control Options | IP Network, USB mouse |
| 7 | Ports | 4 SATA Ports For 4 Channel HD DVR |
|  |  | Each SATA Port Should Support 10 TB HDD or more |
| 8 | Interface | 1 HDMI, 1 VGA, |
| 9 | Encoding Resolution | 5MP, 4MP, 3MP, 1080P, 720P, 960H, D1, CIF, QCIF |
| 10 | IP Camera input | 8+4 channels, each channel up to 6MP |
| 11 | Video Input | 8 Ch PAL (NTSC to PAL Conversion not acceptable) |
| 12 | Display speed | 200 fps for 8 channel (25 fps for each channel) or more |
| 13 | Compression method | AI Coding/H.265+/H.265/H.264+/H.264 |
| 14 | Audio In/Out | Audio input: 4-channel BNC (external) 16-channel BNC (coaxial audio) Audio Ouput: 1-channel BNC |
| 15 | Recording Resolution | 5M-N/4M-N/1080P/1080N/720P/960H/ D1/CIF |
| 16 | Recording rate | 5M-N (1 fps–10 fps); 4M-N/1080p (1 fps–15 fps); 1080N/720p/960H/ D1/CIF (1 fps–25/30 fps) |
| 17 | Recording Interval Prerecord: | 1sec to 30 sec., Post-record: 10sec ~ 5 minutes |
| 18 | Video Encoding Bit Rate Analog video: | 32 kbps–6144 kbps per channel |
| 19 | Image Quality | 6 levels (VBR/ CBR) |
| 20 | Independent Recording setting Per channel | Manual; Schedule (General, Continuous); MD (Video detection: Motion Detection, Video Loss, Tampering); Alarm; Stop |
| 21 | Monitoring | 4 channel real time Monitoring |
|  |  | Single mode & Quad modes |
|  |  | Sequential switching mode |
|  |  | Switching out monitoring |
| 22 | Searching/Playback | Date & Time based |
|  |  | 1 channel searching and all channel simultaneous searching |
|  |  | Time/Date, Alarm, MD and Exact Search (accurate to second) |
| 23 | Object classification | Human/Vehicle secondary recognition for tripwire and intrusion |
| 24 | Smart Motion detection | Up to 8 channels video stream ( analog channel ) SMD Plus |
| 25 | Perimeter Protection | Up to 2 channel video stream ( analog channel ) Perimeter Protection |
| 26 | Back Up facility | USB Device/Network |
| 27 | Network Interface | 1 RJ-45 Port (1000M) |
| 28 | RS-232 | 1 port, for PC Communication & Keyboard |
| 29 | User Interface | On screen graphic User Interface(GUI) |
| 30 | System recovery after power failure | Auto-Rebooting |
| 31 | Alarm input | 16 Alarm Inputs |
| 32 | Alarm Output | 6 Alarm Ouput |
| 33 | USB Interface Minimum | 2 USB ports (USB 2.0) |
| 34 | Serial Interface | RS485 1 port, for PTZ control |
| 35 | Playback Function | Play; Pause; Stop; Rewind; Fast play; Slow Play; Next File; Previous File; Next Camera; Previous Camera; Full Screen; Repeat; Shuffle; Backup Selection; Digital Zoom |
| 36 | Search Mode | Time/Date, Alarm, MD and Exact Search (accurate to second) |
| 37 | Video Alarm Type | Recording, PTZ, Tour, Video Push, Email, FTP, Snapshot, Buzzer and Screen Tips, Motion Detection, Video Loss, Tampering and Diagnosis |
| 38 | Operating Conditions | -10°C ~ +55°C (+14°F ~ +131°F), 0~ 90% RH |
| 39 | Certifications | CE/ FCC/ UL and BIS certification |

16 Channel 8 SATA DVR Technical Specifications:

| S. No. | Particulars | Specifications |
| --- | --- | --- |
| 1 | Operating system | Embedded Linux |
| 2 | CPU | Embedded processor |
| 3 | Compatibility (Mandatory) | Supports HDCVI/AHD/TVI/CVBS/IP video inputs |
| 4 | System Resources | Pentaplex function: recording playback, live view, backup, remote access over IP network |
| 5 | Recorder Control Options | Video Front panel, IR Based remote control, Keyboard (Both RS485 and IP), IP Network, USB mouse |
| 6 | Hard Drive Storage | 8 SATA ports,up to 10 TB, the maximum HDD capacity |
| 7 | Video Output interface | 2 HDMI and 1 VGA 1TV |
| 8 | Encoding Resolution | All channels 5M-N@(1 fps–10 fps); 4M-N@ (1 fps–15 fps); 1080p@(1 fps–15 fps); 1080N@(1 fps– 25/30 fps); 720p@(1 fps–25/30 fps); 960H@(1 fps–25/30 fps); D1@(1 fps–25/30 fps); CIF@(1 fps–25/30 fps) |
| 9 | IP Camera input | Up to 24 channels of IPC access 8 IP channels by default + 16 channels (analog channels are converted to IP channels) Incoming bandwidth: 128 Mbps, recording bandwidth: 128 Mbps and outgoing bandwidth: 128 Mbps |
| 10 | Max Frame Rate support | 1. CVI: 5 MP@25/30 fps; 4 MP@25/30 fps; 1080p@25/30 fps, 720p@50/60 fps; 720p@25/30 fps. 2. TVI: 5 MP@25/30 fps; 4 MP@25/30 fps; 3 MP@25/30 fps; 1080p@25/30 fps; 720p@25/30 fps. 3. AHD: 5 MP@25/30 fps; 4 MP@25/30 fps; 3 MP@25/30 fps; 1080p@25/30 fps; 720p@25/30 fps |
| 11 | Compression method | AI Coding; H.265+; H.265; H.264+; H.264 |
| 12 | Audio Input | 16-channel BNC (external), 16-channel BNC (coaxial audio) |
| 13 | Audio Output | 1-channel BNC |
| 14 | Two Way talk | 1-channel BNC |
| 15 | Recording Resolution | 5M-N@(1 fps–10 fps); 4M-N@ (1 fps–15 fps); 1080p@(1 fps–15 fps); 1080N@(1 fps– 25/30 fps); 720p@(1 fps–25/30 fps); 960H@(1 fps–25/30 fps); D1@(1 fps–25/30 fps); CIF@(1 fps–25/30 fps) Sub stream: D1@(1 fps–15 fps); CIF@(1 fps–15 fps) |
| 16 | Recording Interval Prerecord: | 1sec to 30 sec., Post-record: 10sec ~ 5 minutes |
| 17 | Image Quality | 6 levels (VBR/ CBR) |
| 18 | Independent Recording setting Per channel | Recording, PTZ, Tour, Video Push, Email, FTP, Snapshot, Buzzer and Screen Tips, Motion Detection, Video Loss, Tampering and Diagnosis |
| 19 | Water marking | Support law enforcement grade video authentication, watermarking should be verifiable through player software or proprietary software which shall be provided free of any charges |
| 20 | Artificial Intelligence | Perimeter Protection, Face detection, Face Recognition, SMD Plus |
| 21 | Perimeter Protection AI by DVR | Up to 4 channels video stream (analog channel) Perimeter Protection |
| 22 | SMD Plus by DVR | Up to 16 channels video stream (analog channel) SMD Plus |
| 23 | Face Recognition AI By DVR | Up to 2 channels video stream (analog channel) human face recognition |
| 24 | Network interface | 2 x RJ45 Ports 10/100/1000Mbps |
| 25 | Network Protocols | HTTP; HTTPS; TCP/IP; IPv4; RTSP; UDP; SMTP; NTP; DHCP; DNS; DDNS; P2P, FTP |
| 26 | Interoperability | ONVIF 21.06; CGI |
| 27 | Alarm input/output | Alarm input: 16 channels |
| 28 |  | Alarm Output: 6 channels |
| 29 | USB Interface Minimum | 3 (1 front USB 2.0 port, 2 rear USB 3.0 ports) |
| 30 | Serial Interface | RS-485 : 1 port, for PTZ control RS-232: 1 port, for PC Communication & Keyboard |
| 31 | Operation for Setting and searching | User friendly and all weather condition |
| 32 | Operating Conditions | –10 °C to +55 °C (14 °F to +131 °F) |
| 33 | Certifications | CE, FCC, UL and BIS |

4 Channel 1 SATA ATM DVR with ATM AI Functions:

| Specifications |  |  | Compliance Yes/No |
| --- | --- | --- | --- |
| 1 | Operating system | Embedded RTOS / Linux / Windows |  |
| 2 | CPU | 32 bit DSP or more powerful |  |
| 3 | Analog / HD Camera Input | 4 Channel DVR |  |
| 4 | Video surveillance Hard disk | Surveillance Grade DVR HDD |  |
| 5 | HDD Slots | 1 SATA (upto 10 TB per slot ) |  |
| 6 | Video Input | 4 Ch (Only for ATMs) |  |
| 7 | Display speed | 100 fps for 4 channel (Only for ATMs) (25 fps for each channel) |  |
| 8 | Compression method | H.265 Pro+/H.265 Pro/H.265/H.264+/H.264 |  |
| 9 | Display split screen (Screen mode) | 1,4 for 4 channel,1,9 for 8 Channel &1,16 for 16 channel |  |
| 10 | Encoding Resolution | 4 MP lite@15 fps; 1080p lite/720p lite/VGA/WD1/4CIF/CIF@25 fps (P)/30 fps (N) |  |
| 11 | Audio Input | 1-ch, RCA (2.0 Vp-p, 1 KΩ) |  |
| 12 | Audio Output | 1-ch, RCA (Linear, 1 KΩ) |  |
| 13 | Synchronous Playback | 4 Ch |  |
| 14 | IP Camera Input | 2 Channel in addition to HD channels; 6ch at the expense of HD channels |  |
| 15 | Independent Recording setting Per channel | Continuous, motion detection sensor activated, flexible date /time daily Schedule. |  |
| 16 | Monitoring | 4 channel real time Monitoring. Single mode & 2x2 Mode Sequential switching mode Switching out monitoring |  |
| 17 | Searching / Playback | Date & Time based 1 channel searching and all channel simultaneous searching Replay up to 25 fps in each channel |  |
| 18 | Video Intelligence | Should support Motion detection, Intrusion Detection |  |
| 19 | Smart ATM Features | 1-ch human face mode and 1-ch panel mode for analog channel Human Face Mode : Normal human face, abnormal human face, multiple people |  |
| 20 | Back Up facility | In built & provision for External portable HDD/Pen drive/through USB port. Player software to play back recorded video should automatically be loaded on to the external back up media so that it can be played back on any PC directly. |  |
| 21 | User Interface | On screen graphic User Interface(GUI) |  |
| 22 | System recovery after power failure | Auto Re booting |  |
| 23 | Operation for setting and searching | User friendly |  |
| 24 | Multiple functionality | Should support pentaflex functions to perform all DVR functions at the same time, i.e., record, live view/playback, network (view remotely), administrative and backup. Client software for remote view shall be provided free of any charge if required. |  |
| 25 | Compatibility | Should support all types of cameras |  |
| 26 | Certifications | UL , CE , FCC |  |

4 Channel 2 SATA NVR Technical Speciation’s:

| Sr. No. |  | Parameters | Technical Specifications |
| --- | --- | --- | --- |
| 1 | Video/Audio input | IP video input | 4-Ch |
|  |  | Two-way audio input | 1-ch, RCA (2.0 Vp-p, 1KΩ) |
| 2 | Network | Incoming bandwidth | 80 Mbps Incoming Bandwidth |
|  |  | Outgoing bandwidth | 80 Mbps Outgoing Bandwidth |
|  |  | Third party camera support | ONVF, RTSP |
|  |  | Network Protocols | HTTP; HTTPS; TCP/IP; IPv4/IPv6; UPnP; SNMP; RTSP; UDP; SMTP; NTP; DHCP; DNS; IP Filter; DDNS; FTP; Alarm Server; IP Search (Support IP camera; DVR; NVS; etc.); P2P; auto register |
| 3 | Video/Audio output | Recording resolution | 8MP; 6MP; 5MP; 4MP; 3MP; 1080p; 720p; D1 |
|  |  | Video Compressor | Smart H.265/H.265/Smart H.264/H.264/MJPEG |
|  |  | HDMI & VGA output | HDMI 4K (3840 × 2160)/30Hz, 2K (2560 × 1440)/60Hz, 1920 × 1080/60Hz, 1600 × 1200/60Hz, 1280 × 1024/60Hz, 1280 × 720/60Hz, 1024 × 768/60Hz. |
|  |  |  | VGA : 1920 × 1080/60Hz, 1280 × 1024/60Hz, 1280 × 720/60Hz, 1024 × 768/60Hz |
|  |  | Audio output | 1-ch, RCA (Linear, 1kΩ) |
|  |  | Audio Compression | PCM/G711A/G711U/G726/AAC |
| 4 | Decoding | Capability | 4 × 1080p@30 fps |
| 5 | Hard disk | SATA | Support 2 SATA |
|  |  | Capacity | Up to 10 TB for each disk |
|  |  | Disk Group | Support |
| 6 | Playback | Multi-channel Playback | Should supports up to 4 channels for playback |
|  |  | Record Mode | Manual record; alarm recording; SMD recording; smart recording |
|  |  | Storage | Local HDD; network |
|  |  | Backup | USB storage device |
|  |  | Playback Function | Play/pause/stop/slow/quick/backward/by frame Full screen, backup (video clip/file), partial zoom in, and audio on/off |
| 7 | External interface | HDMI | 1 x HDMI Port |
|  |  | VGA | 1 x VGA Port |
|  |  | Network interface | 1 RJ-45 10 /100 / 1000 Mbps self-adaptive Ethernet interface |
|  |  | USB interface | 2 (2 × back USB2.0) |
| 8 | Others | Power supply | 12V DC / 100~ 240 VAC |
|  |  | Consumption (without hard disk) | ≤ 9.5W |
|  |  | Working temperature | -10 ºC ~ +55 ºC (+14 ºF~ + 131 ºF) |
|  |  | Working humidity | 10 % ~ 90 % |
| 9 | Certification | CE , FCC, BIS |  |

8 Channel 2 SATA NVR Technical Specifications:

| Sr. No. |  | Parameters | Technical Specifications |
| --- | --- | --- | --- |
| 1 | System | Main Processor | Industrial-grade embedded processor |
|  |  | Operating System | Embedded Linux |
|  |  | Operation Interface | Supports configuration through Web; local GUI |
| 2 | Video/Audio input | IP video input | 8-Ch |
|  |  | Third party IP Camera Support | ONVIF(profile T/S/G); CGI; SDK |
|  |  | Two-way audio input | 1-ch, RCA (2.0 Vp-p, 1KΩ) |
| 3 | Network | Incoming bandwidth | 160 Mbps Incoming Bandwidth |
|  |  | Outgoing bandwidth | 160 Mbps Outgoing Bandwidth |
|  |  | ANR | Support |
| 4 |  | Network Protocol | HTTP; HTTPS; TCP/IP; IPv4/IPv6; UPnP; SNMP; RTSP; UDP; SMTP; NTP; DHCP; DNS; IP Filter; DDNS; FTP; Alarm Server; IP Search (Support IP camera; DVR; NVS; etc.); P2P; auto register |
| 5 | Video/Audio output | Recording resolution | 8MP; 6MP; 5MP; 4MP; 3MP; 1080p; 720p; D1 |
|  |  | Decoding Capacity | 8 × 1080p@25/30 fps |
|  |  | Video Compressor | Smart H.265+; Smart H.264+; H.265; H.264; MJPEG |
|  |  | Audio compression | PCM/G711A/G711U/G726/AAC |
|  |  | HDMI & VGA output | HDMI x 1 v and VGA x 1 |
|  |  | Capability | 16-ch @ 1080p |
| 6 | PlayBack | Multi-channel Playback | Up to 8 channels for playback |
|  |  | Record Mode | Manual record; alarm recording; SMD recording; smart recording |
|  |  | Storage | Support Local HDD; network |
| 7 |  | Backup | USB storage device |
|  |  | Playback Function | Play/pause/stop/slow/quick/backward/by frame Full screen, backup (video clip/file), partial zoom in, and audio on/off |
| 8 | Hard disk | SATA | 2 SATA HDD Support |
|  |  | Storage Capacity | Up to 10 TB for each disk |
| 9 | Alarm | General Alarm | Motion detection; privacy masking; video loss; scene changing; PIR alarm; IPC external alarm |
|  |  | Anomaly Alarm | Camera disconnection; storage error; disk full; IP conflict; MAC conflict; login locked; and cybersecurity exception |
|  |  | Intelligent Alarm | Based on Intelligent IP Camera: Face detection, perimeter protection (intrusion and tripwire), IVS, people counting, heat map, and SMD alarms |
|  |  | Alarm Linkag | Recording, snapshot, audio, buzzer, log, preset, and email |
| 10 | External interface | Network interface | 1 × RJ-45, 10/100/1000 Mbps self-adaptive Ethernet port |
|  |  | Alarm | 4 Inputs and 2 Output |
|  |  | USB interface | Should spport 2(1 × front USB2.0; 1 × back USB2.0) |
|  |  | Audio Input | 1 × RCA input |
|  |  | Audio Output | 1 × RCA output |
| 11 | Others | Power supply | DC12 V, 4 A |
|  |  | Consumption (without hard disk) | < 10 W (HDD not included, idling) |
|  |  | Working temperature | –10 °C to +55 °C (+10 °F to +131 °F) |
|  |  | Working humidity | 10%–93% |
| 12 | Certification | CE , FCC and BIS |  |

16 Channel 4 SATA NVR Technical Specifications:

| Sr. No. |  | Parameters | Technical Specifications |
| --- | --- | --- | --- |
| 1 | System | Main Processor | Industrial-grade embedded processor |
|  |  | Operating System | Embedded Linux |
|  |  | Operation Interface | Supports configuration trhough Web; local GUI |
| 2 | Video/Audio input | IP video input | 16-Ch |
|  |  | Two-way audio input | 1-ch, RCA (2.0 Vp-p, 1KΩ) |
| 3 | Network | Incoming bandwidth | 160 Mbps Incoming Bandwidth |
|  |  | Outgoing bandwidth | 160 Mbps Outgoing Bandwidth |
|  |  | ANR | Support |
|  |  | Network Protocol | HTTP; HTTPS; TCP/IP; IPv4/IPv6; UPnP; SNMP; RTSP; UDP; SMTP; NTP; DHCP; DNS; IP Filter; DDNS; FTP; Alarm Server; IP Search (Support IP camera; DVR; NVS; etc.); P2P; auto register |
| 4 | Video/Audio output | Recording resolution | 16MP; 12MP; 8MP; 5MP; 4MP; 3MP; 2MP; 720p; D1 |
|  |  | Decoding Capacity | 16-channel@1080P (30 fps) |
|  |  | Video Compressor | Smart H.265+; Smart H.264+; H.265; H.264; MJPEG |
|  |  | Audio compression | PCM; G.711A; G.711U; G.726 |
|  |  | HDMI & VGA output | HDMI x 1 v and VGA x 1 |
| 5 | Hard disk | SATA | 4 SATA HDD Supports capacity 10 TB Each |
|  |  | eSATA | 1 |
|  |  | Capacity | Up to 10 TB for each disk |
| 7 | Alarm | General Alarm | Motion detection; privacy masking; video loss; scene changing; PIR alarm; IPC external alarm |
|  |  | Anomaly Alarm | Camera disconnection; storage error; disk full; IP conflict; MAC conflict; login locked; and cybersecurity exception |
|  |  | Intelligent Alarm | Face detection; face recognition; perimeter protection; SMD Plus; people counting; stereo analysis; heat map |
|  |  | Alarm Linkag | Record; snapshot; IPC external alarm output; audio; buzzer; log; preset; email |
| 8 | External interface | Network interface | 2 RJ-45 10/100/1000 Mbps self-adaptive Ethernet ports |
|  |  | Alarm | 16 Inputs and 6 Output |
|  |  | USB interface | 1 front USB 2.0 port, 2 rear USB 3.0 ports |
|  |  | RS 232 | 1 channel, used to debug and pass through serial data |
|  |  | RS 485 | Should Support 1 channel, used to control PTZ, and it supports multiple protocols |
|  |  | Audio Input | 1 × RCA input |
|  |  | Audio Output | 1 × RCA output |
| 9 | Others | Power supply | Single, 100 V–240 V AC, 50 Hz–60 Hz |
|  |  | Consumption (without hard disk) | Total output of NVR is ≤ 8 W (without HDD) |
|  |  | Working temperature | –10 °C to +55 °C (+10 °F to +131 °F) |
|  |  | Working humidity | 10%–93% |
| 10 | Certification | CE , FCC and BIS |  |

2 MP IP Dome Camera:

| SR No. | Camera |  |
| --- | --- | --- |
| 1 | Image Sensor | 1/2.8" Progressive Scan CMOS |
| 2 | Min. Illumination | 0.002 Lux@F1.6 (Color, 30 IRE) 0.0002 Lux@F1.6 (B/W, 30 IRE) 0 Lux (Illuminator on) |
| 3 | Shutter Speed | Auto/Manual 1/3 s–1/100,000 s |
| 4 | Day &Night | Auto(ICR)/Color/B/W |
| 5 | Digital Noise Reduction | 3D DNR |
| 6 | WDR | 120dB |
| 7 | HLC | Yes Support |
| 8 | Gain Control | Auto; manual |
| 8 | Angle Adjustment | Pan: 0° to 355°, tilt: 0° to 65°, rotation: 0° to 355° |
|  | Lens |  |
| 9 | Focal length | 2.8 mm; 3.6 mm |
| 10 | Aperture | F1.6 |
| 11 | Focus | Auto/Manual |
| 12 | FOV | 2.8 mm: Horizontal: 107°; Vertical: 56°; Diagonal: 127° 3.6 mm: Horizontal: 88°; Vertical: 44°; Diagonal: 105° |
| 13 | Lens Mount | M12 |
|  | IR |  |
| 14 | IR Range | 30 m (98.43 ft) (IR) |
|  | Video and Compression |  |
| 15 | Streams Capacity | 3 streams |
| 16 | Video Compression | H.265; H.264; H.264B; MJPEG (Only supported by the sub stream) |
|  |  | Smart Codec: Smart H.265+; Smart H.264+ |
| 17 | Video Bit Rate | H.264: 32 kbps–6144 kbps H.265: 12 kbps–6144 kbps |
| 18 | Resolution | 1080p (1920 × 1080); 1.3M (1280 × 960); 720p (1280 × 720); D1 (704 × 576/704 × 480); VGA (640 × 480); CIF (352 × 288/352 × 240) |
|  | Smart Feature-set |  |
| 19 | Region of Interest | Yes (4 areas) |
| 20 | Motion Detection | OFF/ON (4 areas, rectangular) |
| 21 | Smart Events | Intrusion, tripwire |
| 22 | Privacy Masking | 4 areas |
|  | Image |  |
| 23 | Max. Resolution | 1920x1080 |
| 24 | Main Stream Max. Frame Rate | 1920 × 1080@1–25/30 fps |
| 25 | Sub-streamMax. Frame Rate | 704 × 576@1–25 fps , 704 × 480@1–30 fps |
| 26 | Third Stream | CIF@1fps |
| 27 | Image Enhancement | BLC, 3D DNR |
| 28 | Image Setting | Saturation, brightness, contrast, sharpness, AGC, white balance adjustable by client software or web browser |
| 29 | Day/Night Switch | Auto, scheduled |
|  | White Balance | Auto; natural; street lamp; outdoor; manual; regional custom |
|  | Audio |  |
| 30 | Audio Compression | G.711ulaw/G.711alaw/G.722.1/G.726/MP2L2/PCM/MP3/AAC |
| 31 | Audio Bit Rate | 64 Kbps (G.711ulaw/G.711alaw)/16 Kbps (G.722.1)/16 Kbps (G.726)/32 to 192 Kbps (MP2L2)/8 to 320 Kbps (MP3)/16 to 64 Kbps (AAC) |
| 32 | Audio Sampling Rate | 8 kHz/16 kHz/32 kHz/44.1 kHz/48 kHz |
| 33 | Environment Noise Filtering | Yes |
|  | Network |  |
| 34 | Storage | FTP; SFTP; Micro SD card (support max. 256 G); NAS |
| 35 | Alarm Trigger | No SD card; SD card full; SD card error; network disconnection; IP conflict; illegal access; motion detection; video tampering; defocus detection; tripwire; intrusion; voltage detection; security exception |
| 36 | Alarm Linkage | Upload to NAS/memory card/FTP, notify surveillance center, trigger recording, trigger capture, send email |
| 37 | Protocols | IPv4; IPv6; HTTP; TCP; UDP; ARP; RTP; RTSP; RTCP; RTMP;SMTP; FTP; SFTP; DHCP; DNS; DDNS; QoS; UPnP; NTP;Multicast; ICMP; IGMP; NFS; PPPoE; Boujour |
| 38 | MAC | MAC ID of the IP camera should be registred with the OEM |
| 39 | Cyber Security | Video encryption; firmware encryption; configuration encryption; Digest; WSSE; account lockout; security logs; IP/MAC filtering; generation and importing of X.509 certification; syslog; HTTPS; 802.1x; trusted boot; trusted execution; trusted upgrade |
| 40 | Interoperability | ONVIF (Profile S/Profile G/Profile T); CGI; P2P; Milestone |
| 41 | Simultaneous Live View | Up to 6 channels |
|  |  | Up to 32 users |
| 42 | User/Host | 20 (Total bandwidth: 48 M) |
| 43 | Web Browser | IE Chrome Firefox |
|  | Interface |  |
| 44 | Communication Interface | 1 RJ45 10M/100M self-adaptive Ethernet port |
| 45 | Reset Button | Yes |
|  | General |  |
| 46 | Operating Conditions | –40 °C to +60 °C (–40 °F to +140 °F) ≤ 95% |
| 47 | Power Supply | 12 VDC ± 25%, reverse polarity protection |
|  |  | PoE: 802.3af, Class 3 |
| 48 | IP Rating | IP67 |
| 49 | Ingress Protection | IK10 Vandal Proof |
| 50 | Material | Metal |
| 51 | Certifications | UL,CE,FCC |

2 MP IP Bullet Camera:

| SR No. | Camera |  |
| --- | --- | --- |
| 1 | Image Sensor | 1/2.8" Progressive Scan CMOS |
| 2 | Min. Illumination | 0.002 Lux@F1.6 (Color, 30 IRE) 0.0002 Lux@F1.6 (B/W, 30 IRE) 0 Lux (Illuminator on) |
| 3 | Shutter Speed | Auto/Manual 1/3 s–1/100,000 s |
| 4 | Day &Night | Auto(ICR)/Color/B/W |
| 5 | Digital Noise Reduction | 3D DNR |
| 6 | WDR | 120dB |
| 7 | HLC | Yes Support |
| 8 | Gain Control | Auto; manual |
| 9 | Angle Adjustment | Pan: 0° to 360°, tilt: 0° to 90°, rotation: 0° to 360° |
|  | Lens |  |
| 10 | Focal length | 2.8 mm; 3.6 mm |
| 11 | Aperture | F1.6 |
| 12 | Focus | Auto/Manual |
| 13 | FOV | 2.8 mm: Horizontal: 107°; Vertical: 56°; Diagonal: 127° 3.6 mm: Horizontal: 88°; Vertical: 44°; Diagonal: 105° |
| 14 | Lens Mount | M12 |
|  | IR |  |
| 15 | IR Range | 30 m (98.43 ft) (IR) |
|  | Video and Compression |  |
| 16 | Streams Capacity | 3 streams |
| 17 | Video Compression | H.265; H.264; H.264B; MJPEG (Only supported by the sub stream) |
|  |  | Smart Codec: Smart H.265+; Smart H.264+ |
| 18 | Video Bit Rate | H.264: 32 kbps–6144 kbps H.265: 12 kbps–6144 kbps |
| 19 | Resolution | 1080p (1920 × 1080); 1.3M (1280 × 960); 720p (1280 × 720); D1 (704 × 576/704 × 480); VGA (640 × 480); CIF (352 × 288/352 × 240) |
|  | Smart Feature-set |  |
| 20 | Region of Interest | Yes (4 areas) |
| 21 | Motion Detection | OFF/ON (4 areas, rectangular) |
| 22 | Smart Events | Intrusion, tripwire |
| 23 | Privacy Masking | 4 areas |
|  | Image |  |
| 24 | Max. Resolution | 1920x1080 |
| 25 | Main Stream Max. Frame Rate | 1920 × 1080@1–25/30 fps |
| 26 | Sub-streamMax. Frame Rate | 704 × 576@1–25 fps , 704 × 480@1–30 fps |
| 27 | Third Stream | CIF@1fps |
| 28 | Image Enhancement | BLC, 3D DNR |
| 31 | Image Setting | Saturation, brightness, contrast, sharpness, AGC, white balance adjustable by client software or web browser |
| 32 | Day/Night Switch | Auto, scheduled |
|  | White Balance | Auto; natural; street lamp; outdoor; manual; regional custom |
|  | Audio |  |
| 33 | Audio Compression | G.711ulaw/G.711alaw/G.722.1/G.726/MP2L2/PCM/MP3/AAC |
| 34 | Audio Bit Rate | 64 Kbps (G.711ulaw/G.711alaw)/16 Kbps (G.722.1)/16 Kbps (G.726)/32 to 192 Kbps (MP2L2)/8 to 320 Kbps (MP3)/16 to 64 Kbps (AAC) |
| 35 | Audio Sampling Rate | 8 kHz/16 kHz/32 kHz/44.1 kHz/48 kHz |
| 36 | Environment Noise Filtering | Yes |
|  | Network |  |
| 37 | Storage | FTP; SFTP; Micro SD card (support max. 256 G); NAS |
| 38 | Alarm Trigger | No SD card; SD card full; SD card error; network disconnection; IP conflict; illegal access; motion detection; video tampering; defocus detection; tripwire; intrusion; voltage detection; security exception |
| 40 | Alarm Linkage | Upload to NAS/memory card/FTP, notify surveillance center, trigger recording, trigger capture, send email |
| 41 | Protocols | IPv4; IPv6; HTTP; TCP; UDP; ARP; RTP; RTSP; RTCP; RTMP;SMTP; FTP; SFTP; DHCP; DNS; DDNS; QoS; UPnP; NTP;Multicast; ICMP; IGMP; NFS; PPPoE; Boujour |
| 42 | MAC | MAC ID of the IP camera should be registred with the OEM |
| 43 | Cyber Security | Video encryption; firmware encryption; configuration encryption; Digest; WSSE; account lockout; security logs; IP/MAC filtering; generation and importing of X.509 certification; syslog; HTTPS; 802.1x; trusted boot; trusted execution; trusted upgrade |
| 44 | Interoperability | ONVIF (Profile S/Profile G/Profile T); CGI; P2P; Milestone |
| 45 | Simultaneous Live View | Up to 6 channels |
|  |  | Up to 32 users |
| 46 | User/Host | 20 (Total bandwidth: 48 M) |
| 47 | Web Browser | IE Chrome Firefox |
|  | Interface |  |
| 48 | Communication Interface | 1 RJ45 10M/100M self-adaptive Ethernet port |
| 50 | Reset Button | Yes |
|  | General |  |
| 51 | Operating Conditions | –40 °C to +60 °C (–40 °F to +140 °F) |
| 52 | Power Supply | 12 V DC/PoE (802.3af) |
|  |  | PoE: 802.3af, Class 3 |
| 53 | IP Rating | IP67 |
| 54 | Material | Metal |
| 55 | Certifications | UL,CE,FCC |

2 MP IP VF Bullet Camera:

| SR No. | Camera |  |
| --- | --- | --- |
| 1 | Image Sensor | 1/2.8" Progressive Scan CMOS |
| 2 | Min. Illumination | 0.002 Lux@F1.6 (Color, 30 IRE) 0.0002 Lux@F1.6 (B/W, 30 IRE) 0 Lux (Illuminator on) |
| 3 | Shutter Speed | Auto/Manual 1/3 s–1/100,000 s |
| 4 | Day &Night | Auto (ICR)/Color/B/W |
| 5 | Digital Noise Reduction | 3D DNR |
| 6 | WDR | 120dB |
| 7 | HLC | Yes Support |
| 8 | Gain Control | Auto; manual |
| 9 | Angle Adjustment | Pan: 0° to 360°, tilt: 0° to 90°, rotation: 0° to 360° |
|  | Lens |  |
| 10 | Focal length | 2.7 mm–13.5 mm |
| 11 | Aperture | F1.5 |
| 12 | Focus | Auto/ Manual |
| 13 | FOV | Horizontal: 108°–30° Vertical: 56°–17° Diagonal: 131°–35° |
| 14 | Lens Mount | φ14 |
|  | IR |  |
| 15 | IR Range | 60 m (196.85 ft) (IR) |
| 16 | Wavelength | 850 nm |
| 17 | Visual Alarm | Yes |
|  | Video and Compression Standard |  |
|  | Stream capacity | 3 stream |
| 18 | Video Compression | H.265; H.264; H.264B; MJPEG (Only supported by the sub stream) |
| 19 | Smart Codec | Smart H.265+; Smart H.264+ |
| 20 | H.265 Type | Main Profile |
| 21 | Video Bit Rate | H.264: 32 kbps–6144 kbps H.265: 12 kbps–6144 kbps |
| 22 | Resolution | 1080p (1920 × 1080); 1.3M (1280 × 960); 720p (1280 × 720); D1 (704 × 576/704 × 480); VGA (640 × 480); CIF (352 × 288/352 × 240) |
|  | Smart Feature-set |  |
| 23 | Region of Interest | Yes (4 areas) |
| 24 | Motion Detection | OFF/ON (4 areas, rectangular) |
| 25 | Smart Events | Intrusion, tripwire |
| 26 | Privacy Masking | 4 areas |
|  | Image |  |
| 27 | Max. Resolution | 1920x1080 |
| 28 | Main Stream Max. Frame Rate | 50 Hz: up to 25 fps (1920 × 1080, 1280 × 720) |
| 31 | Sub-streamMax. Frame Rate | 50 Hz: 25 fps (704 × 576, 704 × 480) |
| 32 | Third Stream | CIF@1fps |
| 33 | Image Enhancement | BLC, 3D DNR |
| 34 | Image Setting | Saturation, brightness, contrast, sharpness, AGC, white balance adjustable by client software or web browser |
| 35 | Day/Night Switch | Auto, scheduled |
|  | Audio |  |
| 36 | Audio Compression | G.711ulaw/G.711alaw/G.722.1/G.726/MP2L2/PCM/MP3/AAC |
| 37 | Audio Bit Rate | 64 Kbps (G.711ulaw/G.711alaw)/16 Kbps (G.722.1)/16 Kbps (G.726)/32 to 192 Kbps (MP2L2)/8 to 320 Kbps (MP3)/16 to 64 Kbps (AAC) |
| 38 | Audio Sampling Rate | 8 kHz/16 kHz/32 kHz/44.1 kHz/48 kHz |
| 39 | Environment Noise Filtering | Yes |
|  | Network |  |
| 40 | Network Storage | Built-in memory card slot, support microSD card, up to 256 GB |
| 41 | Alarm Trigger | Motion detection, video tampering alarm, network disconnected, IP address conflicted, illegal login, Line crossing detection, intrusion detection |
| 42 | Alarm Linkage | Upload to NAS/memory card/FTP, notify surveillance center, trigger recording, trigger capture, send email, |
| 43 | Protocols | Pv4;IPv6;HTTP;HTTPS;TCP;UDP;ARP;RTP;RTSP;RTCP; RTMP;SMTP;FTP;SFTP;DHCP;DNS;DDNS;QoS;UPnP;NTP; Multicast;ICMP;IGMP;NFS;PPPoE;802.1x;SNMP |
| 44 |  |  |
| 45 | MAC | MAC ID of the IP camera should be registered with the OEM |
| 46 | General Function | One-key reset, anti-flicker, heartbeat, mirror, password protection, privacy mask, watermark |
| 47 | API | ONVIF(Profile S/Profile G);CGI;P2P;Milestone;Genetec |
| 48 | Simultaneous Live View | Up to 6 channels |
|  |  | Up to 32 users |
| 49 | User/Host | 3 levels: Administrator, Operator and User |
| 50 | Web Browser | IIE10+, Chrome57, Firefox52.0+, Safari5.02+ |
|  | Interface |  |
| 51 | Communication Interface | 1 RJ45 10M/100M self-adaptive Ethernet port |
| 52 | Alarm | 1 inputs, 1 outputs |
| 53 | Reset Button | Yes |
| 54 | Audio | 1 input (line in), two-core terminal block, max. input amplitude: 3.3 vpp, input |
|  |  | impedance: 4.7 KΩ, interface type: non-equilibrium; |
|  |  | 1 output (line out), two-core terminal block, max.output amplitude: 3.3 vpp, output |
|  |  | impedance: 100 Ω, interface type: non-equilibrium |
|  | General |  |
| 55 | Operating Conditions | -30 °C to 60 °C (-22 °F to 140°F), humidity: 95% or less (non-condensing) |
| 56 | Power Supply | 12 VDC ± 25%, reverse polarity protection |
|  |  | PoE: 802.3af, Class 3 |
| 57 | Protection Level | IP67 |
| 58 | Material | Metal & Plastic |
| 59 | Certifications | UL,CE,FCC |

2 MP IP VF Dome Camera:

| SR No. | Camera |  |
| --- | --- | --- |
| 1 | Image Sensor | 1/2.8" Progressive Scan CMOS |
| 2 | Min. Illumination | 0.002 Lux@F1.6 (Color, 30 IRE) 0.0002 Lux@F1.6 (B/W, 30 IRE) 0 Lux (Illuminator on) |
| 3 | Shutter Speed | Auto/Manual 1/3 s–1/100,000 s |
| 4 | Day &Night | Auto (ICR)/Color/B/W |
| 5 | Digital Noise Reduction | 3D DNR |
| 6 | WDR | 120dB |
| 7 | HLC | Yes Support |
| 8 | Gain Control | Auto; manual |
| 9 | Angle Adjustment | Pan: 0° to 360°, tilt: 0° to 90°, rotation: 0° to 360° |
|  | Lens |  |
| 10 | Focal length | 2.7 mm–13.5 mm |
| 11 | Aperture | F1.5 |
| 12 | Focus | Auto/ Manual |
| 13 | FOV | Horizontal: 108°–30° Vertical: 56°–17° Diagonal: 131°–35° |
| 14 | Lens Mount | φ14 |
|  | IR |  |
| 15 | IR Range | 40 m (196.85 ft) (IR) |
| 16 | Wavelength | 850 nm |
| 17 | Visual Alarm | Yes |
|  | Video and Compression Standard |  |
| 18 | Stream capacity | 3 stream |
| 19 | Video Compression | H.265; H.264; H.264B; MJPEG (Only supported by the sub stream) |
| 20 | Smart Codec | Smart H.265+; Smart H.264+ |
| 21 | H.265 Type | Main Profile |
| 22 | Video Bit Rate | H.264: 32 kbps–6144 kbps H.265: 12 kbps–6144 kbps |
| 23 | Resolution | 1080p (1920 × 1080); 1.3M (1280 × 960); 720p (1280 × 720); D1 (704 × 576/704 × 480); VGA (640 × 480); CIF (352 × 288/352 × 240) |
|  | Smart Feature-set |  |
| 24 | Region of Interest | Yes (4 areas) |
| 25 | Motion Detection | OFF/ON (4 areas, rectangular) |
| 26 | Smart Events | Intrusion, tripwire |
| 27 | Privacy Masking | 4 areas |
|  | Image |  |
| 28 | Max. Resolution | 1920x1080 |
| 29 | Main Stream Max. Frame Rate | 50 Hz: up to 25 fps (1920 × 1080, 1280 × 720) |
| 30 | Sub-streamMax. Frame Rate | 50 Hz: 25 fps (704 × 576, 704 × 480) |
| 31 | Third Stream | CIF@1fps |
| 32 | Image Enhancement | BLC, 3D DNR |
| 33 | Image Setting | Saturation, brightness, contrast, sharpness, AGC, white balance adjustable by client software or web browser |
| 34 | Day/Night Switch | Auto, scheduled |
|  | Audio |  |
| 35 | Audio Compression | G.711ulaw/G.711alaw/G.722.1/G.726/MP2L2/PCM/MP3/AAC |
| 36 | Audio Bit Rate | 64 Kbps (G.711ulaw/G.711alaw)/16 Kbps (G.722.1)/16 Kbps (G.726)/32 to 192 Kbps (MP2L2)/8 to 320 Kbps (MP3)/16 to 64 Kbps (AAC) |
| 37 | Audio Sampling Rate | 8 kHz/16 kHz/32 kHz/44.1 kHz/48 kHz |
| 38 | Environment Noise Filtering | Yes |
|  | Network |  |
| 39 | Network Storage | Built-in memory card slot, support microSD card, up to 256 GB |
| 40 | Alarm Trigger | Motion detection, video tampering alarm, network disconnected, IP address conflicted, illegal login, Line crossing detection, intrusion detection |
| 41 | Alarm Linkage | Upload to NAS/memory card/FTP, notify surveillance center, trigger recording, trigger capture, send email, |
|  | Protocols | Pv4;IPv6;HTTP;HTTPS;TCP;UDP;ARP;RTP;RTSP;RTCP; RTMP;SMTP;FTP;SFTP;DHCP;DNS;DDNS;QoS;UPnP;NTP; Multicast;ICMP;IGMP;NFS;PPPoE;802.1x;SNMP |
| 42 |  |  |
| 43 | MAC | MAC ID of the IP camera should be registered with the OEM |
| 44 | General Function | One-key reset, anti-flicker, heartbeat, mirror, password protection, privacy mask, watermark |
| 45 | API | ONVIF(Profile S/Profile G);CGI;P2P;Milestone;Genetec |
| 46 | Simultaneous Live View | Up to 6 channels |
|  |  | Up to 32 users |
| 47 | User/Host | 3 levels: Administrator, Operator and User |
| 48 | Web Browser | IIE10+, Chrome57, Firefox52.0+, Safari5.02+ |
|  | Interface |  |
| 49 | Communication Interface | 1 RJ45 10M/100M self-adaptive Ethernet port |
| 50 | Alarm | 1 inputs, 1 outputs |
| 51 | Reset Button | Yes |
| 52 | Audio | 1 input (line in), two-core terminal block, max. input amplitude: 3.3 vpp, input |
|  |  | impedance: 4.7 KΩ, interface type: non-equilibrium; |
|  |  | 1 output (line out), two-core terminal block, max.output amplitude: 3.3 vpp, output |
|  |  | impedance: 100 Ω, interface type: non-equilibrium |
|  | General |  |
| 53 | Operating Conditions | -30 °C to 60 °C (-22 °F to 140°F), humidity: 95% or less (non-condensing) |
|  | Power Supply | 12 VDC ± 25%, reverse polarity protection |
| 54 |  | PoE: 802.3af, Class 3 |
| 55 | Protection Level | IP67 and IK10 Protection |
| 56 | Material | Metal & Plastic |
| 57 | Certifications | UL,CE,FCC |

2 MP HD Fixed Bullet Camera:

| Sr. No | Parameters | Specification |
| --- | --- | --- |
| 1 | Image Sensor | 1/2.8" CMOS Progressive CMOS Sensor |
| 2 | Effective Pixels | 1937(H)×1097(V), 2.1M |
| 3 | Electronic Shutter Speed | PAL: 1/4s~1/100000s NTSC: 1/3s~1/100000s |
| 4 | Mini. Illumination | 0.005Lux/F1.6, 30IRE, 0Lux IR on |
| 5 | S/N Ratio | More than 65dB |
| 6 | IR Distance | Up to 30m (98feet) |
| 7 | Day/Night | Auto (ICR)/Manual |
| 8 | IR On/Off Control | Auto / Manual |
| 9 | IR LEDs | 18 |
| 10 | Lens Type | Fixed Lens/Fixed Iris |
| 11 | Mount Type | Board-in |
| 12 | Focal Length | 3.6mm (2.8mm Optional) |
| 13 | Max Aperture | F1.6 |
| 14 | HD Technology | AHD, CVI, TVI, CVBS |
| 15 | Signal System | PAL |
| 16 | Video Output | 1080P or higher @ 25 fps |
| 17 | Display Resolution | 1080P or higher @ 25 fps |
| 18 | Recording Resolution | 1080P or higher @ 25 fps |
| 19 | Frame Rate | 25/30fps@1080P, 25/30fps@720P |
| 20 | Sync System | Internal |
| 21 | White Balance | Automatic |
| 22 | Shutter | Automatic |
| 23 | Auto Gain Control (AGC) | Automatic |
| 24 | Noise Reduction (NR) | 3DNR |
| 25 | Functions | Brightness, Sharpness, 3D DNR, Smart light |
| 26 | Angle of View | H: 87° (110.5°/52.9° |
| 27 | WDR | 120 db WDR ( DWDR not acceptable) |
| 28 | BLC Mode | Support |
| 29 | HLC Mode | Support |
| 30 | Smart IR | Support Auto/Manual |
| 31 | Input Voltage | 12V DC ±25% |
| 32 | IP Rating | IP67 |
| 33 | Operation Temp | -40°C ~ +60°C (-40°F ~ +140°F) |
| 34 | Storage Humidity | Less than 90% RH |
| 35 | Casing | Aluminium |
| 36 | Certification | CE, FCC and UL |

2 MP HD Fixed Dome Camera:

| S.No | Parameters | Specification |
| --- | --- | --- |
| 1 | Image Sensor | 1/2.8" Progressive CMOS sensor |
| 2 | Effective Pixels | 1937(H)×1097(V), 2.1MP |
| 3 | Electronic Shutter Speed | PAL: 1/4s~1/100000s NTSC: 1/3s~1/100000s |
| 4 | Mini. Illumination | 0.005Lux/F1.6, 30IRE, 0Lux IR on |
| 5 | S/N Ratio | More than 65dB |
| 6 | IR Distance | Up to 30m (98feet) |
| 7 | Day/Night | Auto (ICR)/ Manual |
| 8 | IR On/Off Control | Auto / Manual |
| 9 | IR LEDs | 12 |
| 10 | Lens Type | Fixed Lens/ Fixed Iris |
| 11 | Mount Type | Board-in |
| 12 | Focal Length | 3.6 mm (2.8 mm Optional) |
| 13 | Max Aperture | F1.6 |
| 14 | HD Technology | AHD, CVI, TVI, CVBS |
| 15 | Signal System | PAL |
| 16 | Video Output | 1080P or higher @ 25 fps |
| 17 | Display Resolution | 1080P or higher @ 25 fps |
| 18 | Recording Resolution | 1080P or higher @ 25 fps |
| 19 | Frame Rate | 25/30fps@1080P, 25/30fps@720P |
| 20 | Sync System | Internal |
| 21 | White Balance | Automatic |
| 22 | Shutter | Automatic |
| 23 | Auto Gain Control (AGC) | Automatic |
| 24 | Noise Reduction (NR) | 3DNR |
| 25 | Functions | Brightness, Sharpness, 3D DNR, Smart light |
| 26 | Angle of View | H: 87° (110.5°/52.9°) |
| 27 | WDR | 120 db WDR ( DWDR not acceptable) |
| 28 | BLC Mode | Support |
| 29 | HLC Mode | Support Auto/Manual |
| 30 | Smart IR | Auto/Manual |
| 31 | Input Voltage | 12V DC ±25% |
| 32 | IP Rating | IP67 |
| 33 | Vandal Resistance | IK10 |
| 34 | Operation Temp | -40°C ~ +60°C (-40°F ~ +140°F) |
| 35 | Storage Humidity | Less than 90% RH |
| 36 | Casing | Aluminium |
| 37 | Certification | UL ,CE,FCC |

2 MP HD VF Bullet Camera:

| S.No | Parameters | Specification |
| --- | --- | --- |
| 1 | Image Sensor | 1/2.8" CMOS Progressive CMOS Sensor |
| 2 | Effective Pixels | 1937(H)×1097(V), 2.1M |
| 3 | Electronic Shutter Speed | PAL: 1/4s~1/100000s NTSC: 1/3s~1/100000s |
| 4 | Mini. Illumination | 0.005Lux/F1.3, 30IRE, 0Lux IR on |
| 5 | S/N Ratio | More than 65dB |
| 6 | IR Distance | Up to 60m (197feet) |
| 7 | Day/Night | Auto (ICR)/Manual |
| 8 | IR On/Off Control | Auto / Manual |
| 9 | IR LEDs | 4 |
| 10 | Lens Type | Vari-focal lens / Fixed iris |
| 11 | Mount Type | Board-in |
| 12 | Focal Length | 2.7-13.5mm |
| 13 | Max Aperture | F1.3 |
| 14 | HD Technology | AHD, CVI, TVI, CVBS |
| 15 | Signal System | PAL |
| 16 | Video Output | 1080P or higher @ 25 fps |
| 17 | Display Resolution | 1080P or higher @ 25 fps |
| 18 | Recording Resolution | 1080P or higher @ 25 fps |
| 19 | Frame Rate | 25/30fps@1080P, 25/30fps@720P |
| 20 | Sync System | Internal |
| 21 | White Balance | Auto/Manual |
| 22 | Shutter | Automatic |
| 23 | Auto Gain Control (AGC) | Automatic |
| 24 | Noise Reduction (NR) | 2D/3D |
| 25 | Functions | Brightness, Sharpness, 3D DNR, Smart light |
| 26 | Angle of View | H: 102°~29°(40°~17°) |
| 27 | WDR | 120 db WDR ( DWDR not acceptable) |
| 28 | BLC Mode | Support |
| 29 | HLC Mode | Support |
| 30 | Smart IR | Support Auto/Manual |
| 31 | Input Voltage | 12V DC ±25% |
| 32 | IP Rating | IP67 |
| 33 | Operation Temp | -40°C ~ +60°C (-40°F ~ +140°F) |
| 34 | Storage Humidity | Less than 90% RH |
| 35 | Casing | Aluminium |
| 36 | Certification | CE, FCC and UL |

2 MP HD VF Dome Camera:

| Sr.No | Parameters | Specification |
| --- | --- | --- |
| 1 | Image Sensor | 1/2.8" CMOS Progressive CMOS Sensor |
| 2 | Effective Pixels | 1937(H)×1097(V), 2.1M |
| 3 | Electronic Shutter Speed | PAL: 1/4s~1/100000s NTSC: 1/3s~1/100000s |
| 4 | Mini. Illumination | 0.005Lux/F1.3, 30IRE, 0Lux IR on |
| 5 | S/N Ratio | More than 65dB |
| 6 | IR Distance | Up to 30m (98feet) |
| 7 | Day/Night | Auto (ICR)/Manual |
| 8 | IR On/Off Control | Auto / Manual |
| 9 | IR LEDs | 2 |
| 10 | Lens Type | Vari-focal lens / Fixed iris |
| 11 | Mount Type | Board-in |
| 12 | Focal Length | 2.7-13.5mm |
| 13 | Max Aperture | F1.3 |
| 14 | HD Technology | AHD, CVI, TVI, CVBS |
| 15 | Signal System | PAL |
| 16 | Video Output | 1080P or higher @ 25 fps |
| 17 | Display Resolution | 1080P or higher @ 25 fps |
| 18 | Recording Resolution | 1080P or higher @ 25 fps |
| 19 | Frame Rate | 25/30fps@1080P, 25/30fps@720P |
| 20 | Sync System | Internal |
| 21 | White Balance | Auto/Manual |
| 22 | Shutter | Automatic |
| 23 | Auto Gain Control (AGC) | Automatic |
| 24 | Noise Reduction (NR) | 2D/3D |
| 25 | Functions | Brightness, Sharpness, 3D DNR, Smart light |
| 26 | Angle of View | H: 102°~29°(40°~17°) |
| 27 | WDR | 120 db WDR ( DWDR not acceptable) |
| 28 | BLC Mode | Support |
| 29 | HLC Mode | Support |
| 30 | Smart IR | Support Auto/Manual |
| 31 | Input Voltage | 12V DC ±25% |
| 32 | Up to 30m (98feet) | IP67 & IK10 |
| 33 | Operation Temp | -40°C ~ +60°C (-40°F ~ +140°F) |
| 34 | Storage Humidity | Less than 90% RH |
| 35 | Casing | Aluminium |
| 36 | Certification | CE, FCC and UL |

2 MP HD Pinhole camera:

| Sr No | Feature | specification | Compliance Yes/No |
| --- | --- | --- | --- |
| 1 | Image Sensor | 1/3'' CMOS /CCD or better |  |
| 2 | Effective pixels | 1920(H)x1080(V)) |  |
| 3 | Resolution | 1920(H)x1080(V)) |  |
| 4 | Sync system | Internal |  |
| 5 | Min. illumination | Colour-0.05Lux B/W 0.005Lux or better |  |
| 6 | S/N ratio | 52 dB |  |
| 7 | Electronic shutter | Auto, 1/50 , 1/100,000 sec |  |
| 8 | Lens | 4 mm or <4mm |  |
| 9 | WDR | 120 db WDR |  |
| 10 | BLC | On/Off ( Auto) |  |
| 11 | Motion detection | Yes |  |
| 12 | Gamma | Auto/manual |  |
| 13 | White balance | Set/Manual/Indoor/Outdoor/ Auto |  |
| 14 | AGC | Off/Low/Middle/High |  |
| 15 | Day & Night | Color/B/W/Auto level adjust |  |
| 16 | Video output | 1.00V p-p Composite Video Output, 75 Ohm |  |
| 17 | Power supply | DC12V |  |
| 18 | Power consumption | 3 Watts max |  |
| 19 | Certification | UL ,CE,FCC & BIS |  |

4 Port POE Switch:

| S.No | Parameters | Specification |
| --- | --- | --- |
| 1 | Data Transmission Por | 4 PoE ports with 10/100 Mbps Base-TX 2 uplink ports with 10/100Mbps Base-TX |
| 2 | PoE Budget | Port1-4 ≤30W, whole PoE budget ≤36W |
| 3 | PoE Standard | IEEE802.3af, IEEE802.3a |
| 4 | PoE Pin Assignment | PoE: 1, 2, 4, 5(V+), 3, 6, 7, 8(V-) |
| 5 | Switching Capacity | 1.2 Gbps |
| 6 | Packet Forwarding Rat | 0.89 Mpps |
| 7 | MAC Table Size | 2K |
| 8 | Jumbo Frame | 2048 Byate |
| 9 | Communication Standard | IEEE 802.3, IEEE 802.3u, IEEE802.3, IEEE802.3X |
| 10 | ESD | Air discharge: 8kV Contact discharge: 6kV |
| 11 | Thunderproof | Common mode: 6kV Differential mode: 4kV |
| 12 | Power Supply | AC100–AC240V |
| 13 | Operating Temperature | -10°C to +55°C (-14°F–131°F) |
| 14 | Operating Humidity | 10%–90% |
| 15 | Power Consumption | Idling: 0.5W Full load: 36W |
| 16 | Certificates | CE, FCC |

8 Port POE Switch:

| Sr, No | Parameters | Specifications |
| --- | --- | --- |
| 1 | Data Transmission Port | Yes8 PoE ports with 10/100 Mbps Base-TX 2 uplink ports with 10/100Mbps Base-TX |
| 2 | PoE Budget | Port1-8 ≤30W, whole PoE budget ≤65W |
| 3 | PoE Standard | IEEE802.3af, IEEE802.3at |
| 4 | PoE Pin Assignment | PoE: 1, 2, 4, 5(V+), 3, 6, 7, 8(V-) |
| 5 | Switching Capacity | 2.0 Gbps |
| 6 | Packet Forwarding Rate | 1.49 Mpps |
| 7 | MAC Table Size | 2 K |
| 8 | Jumbo Frame | 2048 byte |
| 9 | Communication Standard | IEEE 802.3, IEEE 802.3u, IEEE 802.3X |
| 10 | ESD | Air discharge: 8kV Contact discharge: 6kV |
| 11 | Thunderproof | Common mode: 6kV Differential mode: 4kV |
| 12 | Power Supply | AC100–AC240V |
| 13 | Operating Temperature | -10°C to +55°C (-14°F–131°F) |
| 14 | Operating Humidity | 10%–90% |
| 15 | Power Consumption | Idling: 1.0W Full load: 65W |
| 16 | Certifications | CE, FCC |

16 Port POE Switch:

| Sr, No | Parameters | Specifications | Compliance |
| --- | --- | --- | --- |
| 1 | Layer 2 | Yes |  |
| 2 | Switching Capacity | 7.2 Gbps |  |
| 3 | Packet Forwarding Rate | 5.3568 Mpps |  |
| 4 | Packet Buffer Memory | 4 Mbit |  |
| 5 | MAC Table Size | 8K |  |
| 6 | Standards Compliance | IEEE802.3, IEEE802.3u, IEEE802.3x, IEEE802.3ab, IEEE802.3z |  |
| 7 | Optical Port Rate | 1 Gbp |  |
| 8 | Number of Bussiness Card Slots | Port 1–16: 16 × RJ45 10/100 Mbps Port 17–18: 2 × RJ45 10/100/1000 Mbps (uplink) Port 17–18: 2 × SFP 1000 Mbps (uplink) (combo) |  |
| 9 | Power Supply | 100V AC–240V AC |  |
| 10 | Operating Temperature | –10°C to 55°C (14°F to 131°F) |  |
| 11 | Operating Humidity | 10%–90% |  |
| 12 | Storage Humidity | 5%–95% |  |
| 13 | Power Consumption | Idling: 5W Full load: 135W |  |
| 14 | PoE Standard | IEEE802.3af, IEEE802.3at, Hi-PoE |  |
| 15 | PoE Budget | Port 1–2≤60W, Port 3–16≤30W, total≤135W |  |
| 16 | PoE Pin Assignment | PoE/Hi-PoE: 1, 2, 4, 5 (V+), 3, 6, 7, 8 (V-) |  |
| 17 | Long Distance PoE Transmission | Yes |  |
| 18 | VLAN | Port Isolation |  |
| 19 | QoS | Yes |  |
| 20 | ESD | Air discharge: 8kV Contact discharge: 6kV |  |
| 21 | Thunderproof | Common mode: 4kV Differential mode: 2kV |  |
| 22 | Certification | CE, FCC |  |